 

vendredi 5 décembre 2008

11:22

 

[image: image1.jpg]PAGE 1 - b3bg3jp0 - removing - refitting : front suspension strut

removing - refitting : front suspension strut

URGENT : Ensure that the precautions relating to safety and cleanliness are adhered to.

1 Recommended tools

E5ap1gke.
-Fig.:1-

[ 1] wiper arm extractor type FACOM U.14A.

ESap05xc.
-Fig.:2-



[image: image2.jpg]PAGE 2 - b3bg3jp0 - removing - refitting : front suspension strut

[2]tool 9501-T L (tool box 9501-T ).

2 Removing

Lift and support the vehicle with the front wheels suspended.

B3bp149c.
-Fig.:3-

Remove
o the front wheels
o the nut (1) (anti-roll bar link rod)

o the screw ( 2) (fixing the suspension strut to the pivot)

The screw ( 3) (fixing of the brake hose support on the pivot).



[image: image3.jpg]PAGE 3 - b3bg3jp0 - removing - refitting : front suspension strut

B3bp124c.
-Fig.:4-

Position the tool [ 2 ] in the slot in the pivot.
Turn a quarter turn to open the pivot jaws.

Remove the pivot from the suspension strut.

CAUTION : While removing the suspension strut, make sure that the driveshaft remains in the differential.

\

B3bpl4ic.
-Fig.:5-



[image: image4.jpg]PAGE 4 - b3bg3jp0 - removing - refitting : front suspension strut

Remove

« the windscreen wiper arms using tool [ 1]
« the scuttle grille (see the relevant operation)

Remove screws (4 ).

URGENT : Do not hold the suspension strut by the upper part.

Remove the suspension strut.

3 Refitting

CAUTION : Always fit new Nylstop nuts.

URGENT : Systematically replace the screw ( 3 ) (fixing of the brake hose support on the pivot).

Insert the suspension strut into the wheel arch.

Place the suspension strut in the jaws of the pivot.

B3bp090c.
-Fig.:6-

URGENT : Ensure that the suspension strut is correctly located in the pivot.

The suspension strut is correctly positioned in the pivot by locating the boss " a " in the groove "b ".

The bosses " ¢ " act as a stop.



[image: image5.jpg]PAGE 5 - b3bg3jp0 - removing - refitting : front suspension strut

Remove tool [2].

Raise the suspension strut and pivot assembly using a jack.

Fit

« the screw ( 2) (tightening torque 4,5 + 0,4 m.daN)

o the screws (4) (tightening torque 2,5 + 0,3 m.daN)

o the nut ( 1) (tightening torque 3,7 + 0,3 m.daN)

o the screw ( 3) (new) (tightening torque 2,7 + 0,2 m.daN)
o the front wheels

« the scuttle grille (see the relevant operation)

« the windscreen wiper arms

Return the vehicle to its wheels.




 

 

 

vendredi 5 décembre 2008

11:23

 
[image: image6.jpg]PAGE 1 - b3bg3Ip0 - removing - refitting : front damper (on the work bench)

removing - refitting : front damper (on the work bench)

1 Recommended tools

E5xp07ec.
-Fig.:1-

[ 11spring compressor 4137-T M90.
[2] spring cups 4137-T M2.
[ 2F ] fixed cup.

[2C ] sliding cup.

2 Preliminary operation

Remove the suspension strut (see the relevant operation).

3 Removing



[image: image7.jpg]PAGE 2 - b3bg3Ip0 - removing - refitting : front damper (on the work bench)

B3bp052c.
-Fig.:2-

Place the suspension strut fitted with the compressor [ 1] and spring cups [ 2] in a vice using a FACOM D22 tool.

Compress the spring ( 8 ) using tool [ 1], [2].

B3bp108d.
-Fig.:3-

Remove

e thenut (1)



[image: image8.jpg]PAGE 3 - b3bg3Ip0 - removing - refitting : front damper (on the work bench)

* washer (2)

e seal (3)

o damper support (4)

e the ball thrust bearing (5)
thrust cup (6)

e washer (7)

Release the spring ( 8 ) using tool [ 1], [2].

Remove
o the spring (8)

o the suspension strut gaiter (9)
« the suspension strut (10)

4 Refitting

URGENT : Systematically change the nut (1).

B3bp052c.
-Fig.: 4 -

Fit

« the suspension strut (10)
« the suspension strut gaiter (9)
o the spring (8)

Compress the spring ( 8 ) using tool [ 1], [2].
Fit



[image: image9.jpg]PAGE 4 - b3bg3Ip0 - removing - refitting : front damper (on the work bench)

washer (7)

thrust cup (6)

the ball thrust bearing (5)
damper support (4 )

seal (3)

washer (2)

thenut (1)

Tighten nut (1) to 4,5+ 0,4 m.daN.

Release the spring using tool [ 1], [2].
N.B. : Check that the spring is correctly positioned on the cup.

Remove tools[1]and [2].

5 Complementary operation

Fit the suspension strut (see the relevant operation).




